Switch commands -

Switch to privileged Exec mode

>Enable

Switch to global configuration mode

#Config t

Assign Switch Hostname

(config)#Hostname S1

Enables Password Encryption

(config)#Service Password-Encryption

Assigns class as the scret password for privileged EXEC mode Access

(config)#Enable secret class

Prevent unwatned DNS lookups.

(config)#No ip domain-lookup

Configure MOTD Banner

(config)#banner motd #Unauthorized access is strictly prohibited#

Verify access settings by moving exiting to the lowest level and enabling back up and make sure you see the motd and have to enter correct passwords.

exit(as many times as needed to get down to lowest level)

Then enable and config t to check if you need passwords.

Creates new vlan 99 on switch

(config)#vlan 99 (then exit back out)

Configure interface

(config)#Interface vlan99

Set ip address of a interface

(config-if)#ip address 192.168.1.2 255.255.255.0

enables interface as soon as there is any connections.

(config-if)#no shutdown

Assign ports to vlan

(config)#interface range f0/1- 24, g0/1 -2

(config)#switchport access vlan 99

See vlans,name,status and ports

Show vlan brief

Set default gateway

(config)#ip default-gateway 192.168.1.1

Turn on password for console port access

(config)#line con 0

(config-line)#password cisco

(config-line)#login

(config-line)#logging synchronous

Allow telnet access with password

(config)#Line vty 0 15

(config-line)#password cisco

(config-line)#login

(config-line)#end

Show interface (vlan)setting

(config)#Show interface vlan 99

Save running configuration

#Copy running-config startup-config

Show mac address learned by switch

#Show mac address-table

Clear mac address table

#Clear mac address0table dynamic

Setup static mac address on interface

(config)#mac address-table static 0050.56BE.6C89 vlan 99 interface fastethernet 0/6

Remove static entry

(config)#no mac address-table static 0050.56BE.6C89 vlan 99 interface fastethernet 0/6

Clearing router and restarting

>Enable

#Erase startup-config

#reload

(may ask you to save, say no, say no to intial configuration dialog, say yes to terminate autoinstall)

Clear and reload switch

>enable

#Delete vlan.dat

#Erase startup-config

#reload

(may be asked to save say no, say no to inital confiuration

Switch Trunking

Interface f0/1

Switchport mode dynamic desirable

Set native Vlan

Switchport trunk native vlan vlan_id

Change sdm(before setting up dchp)

Sdm prefer lanbase-routing

Router stuff(mostly similar to switch)

Loopback interface

Interface Lo1

Ip address as alwaysh

Static route

Ip route 0.0.0.0 0.0.0.0 {ip address or exit interface}

PC Settings

Change ip address

goto go to desktop - ip configuration

Switch ALWAYS DO THESE

Enable

Config t

Hostname s1

No ip domain-lookup

service password-encryption

Enable secret class(prileged exec password)

Line con 0 (console port access)

Password cisco

Login

Logging synchronous

Exit

Line vty 0 15 (telnet access)

Password cisco

login

End

Config t

#banner motd #UNAUHORIZED ACCESS IS STRICTLY PROHIBITED#

Vlan 99

Name (name here if needed)

Exit

Interface vlan 99

ip address 172.16.99.11 255.255.255.0

No shutdown

end

Ip default-gateway 172.16.99.1

Ip domain-name CCNA-Lab.com

Username admin privilege 15 secret sshadmin

Line vty 0 15

Transport input ssh

Login local

Exit

Crypto key generate rsa

1024 when asking how many bits

Ip ssh version 2

Ip ssh time-out 75

Ip shh authentication-retries 2

End

(SHUTDOWN UNUSED PORTS)

Interface range f0/1 - 4

shutdown

Non ip http server

Enable security

Itnerface f0/5

Shutdown

Switchport mdoe access

Switchport port-security

Switchport port-security mac-address xxxx.xxxx.xxxx(find this from using show interface on the other device it connects to and using the interface it connects to)

Router ALWAYS DO THESE

Enable

Config t

Hostname R1

No ip domain-lookup

Interface G0/1

ip address 172.16.99.1 255.255.255.0

no shutdown

Exit

Service password-encryption

Enable secret class(prileged exec assword)

Line con 0

Password cisco

Login

Logging synchronous

Exit

Line vty 0 15

Password cisco

login

End
Config t

#banner motd #UNAUHORIZED ACCESS IS STRICTLY PROHIBITED#

Copy running-config startup-config

|  | Addresses | Hosts | Netmask | Amouint of a class C |
| --- | --- | --- | --- | --- |
| / 30 | 4 | 2 | 255.255.255.252 | 1 / 64 |
| / 29 | 8 | 6 | 255.255.255.248 | 1 / 32 |
| / 28 | 16 | 14 | 255.255.255.240 | 1 / 16 |
| / 27 | 32 | 30 | 255.255.255.224 | 1 / 8 |
| / 26 | 64 | 62 | 255.255.255.192 | 1 / 4 |
| / 25 | 128 | 126 | 255.255.255.128 | 1 / 2 |
| / 24 | 256 | 254 | 255.255.255.0 | 1 |
| / 23 | 512 | 510 | 255.255.254.0 | 2 |
| / 22 | 1024 | 1022 | 255.255.252.0 | 4 |
| / 21 | 2048 | 2046 | 255.255.248.0 | 8 |
| / 20 | 4096 | 4094 | 255.255.240.0 | 16 |
| / 19 | 8192 | 8190 | 255.255.224.0 | 32 |
| / 18 | 16384 | 16382 | 255.255.192.0 | 64 |
| / 17 | 32768 | 32766 | 255.255.128.0 | 128 |
| / 16 | 65536 | 65534 | 255.255.0.0 | 256 |

Practice practical commands

Switch

service password-encryption

ip ssh version 2

ip ssh authentication-retries 2

ip ssh time-out 60

ip domain-name cisco.com

switchport access vlan 30

switchport mode access

switchport port-security

switchport port-security maximum 2

switchport port-security mac-address sticky

switchport port-security violation restrict

switchport trunk native vlan 45

switchport mode trunk

interface Vlan60

mac-address 0001.c9aa.4801

ip address 192.168.45.34 255.255.255.240

ip default-gateway 192.168.45.33

line vty 0 4

login local

transport input ssh

line vty 5 15

login local

transport input ssh

Router

ip dhcp pool LAN

network 192.168.45.128 255.255.255.192

default-router 192.168.45.129

ip cef

no ipv6 cef

no ip address

duplex auto

speed auto

no shutdown

interface GigabitEthernet0/0.15

encapsulation dot1Q 15

ip address 192.168.45.65 255.255.255.192

ip nat inside

interface Serial0/0/0

ip address 192.168.45.1 255.255.255.252

ip nat inside

clock rate 2000000

interface Serial0/0/1

ip address 192.168.45.10 255.255.255.252

ip nat inside

interface Serial0/1/0

ip address 128.107.50.66 255.255.255.252

ip nat outside

router rip

version 2

passive-interface GigabitEthernet0/0

passive-interface GigabitEthernet0/0.15

passive-interface GigabitEthernet0/0.30

passive-interface GigabitEthernet0/0.45

passive-interface GigabitEthernet0/0.60

network 192.168.45.0

ip nat pool JIMJ 64.100.32.56 64.100.32.57 netmask 255.255.255.252

ip nat inside source list 1 pool JIMJ

ip nat inside source static 192.168.45.66 64.100.32.58

ip classless

ip route 0.0.0.0 0.0.0.0 Serial0/1/0

access-list 1 permit 192.168.45.0 0.0.0.255

Own Order

Switch First

Service password-encryption

Ip domain-name cisco.com

Username CAdmin privilege 15 secret itsasecret

Crypto key generate rsa

1024

Ip ssh version 2

Ip ssh time-out 60

Ip ssh authentication-retries 2

line vty 0 4

login local

transport input ssh

line vty 5 15

login local

transport input ssh

Ip default-gateway

Vlan 15 -name servers - exit

Interface vlan 15 - Ip address 192.168.45.66 255.255.255.192

Vlan 30 - name PCs - exit

Interface vlan 30 - ip address 192.168.45.130 255.255.255.192

Vlan 45 - name Native - exit

Interface vlan 45 - ip address 192.168.45.18 255.255.255.240

Vlan 60 - name Management - exit

Interface vlan 60 - ip address 192.168.45.34 255.255.255.240

Interface range fa0/1 - 10 -  switchport access vlan 30

Interface range fa0/11-20 - switchport access vlan 15

Interface range fa0/1-20 switchport mode access

Interface g0/1 - switchport trunk native vlan 45 - switchport mode trunk

Interface fa0/1 - switchport mode access - switchport port-security - switchport port-security maximum 2 - switchport port-security mac-address sticky - switchport port-security violation restrict

Interface range fa0/21-24 - shutdown

Interface g0/2 shutdown

ip default-gateway 192.168.45.33

Router

Interface g0/0.15 - Encapsulation dot1Q 15 - Ip address 192.168.45.65 255.255.255.192 - ip nat inside

Interface g0/0.30 - Encapsulation dot1Q 30 - ip address 192.168.45.129 255.255.255.192- ip nat inside

Interface g0/0.45 - encapsulation dot1q 45 native - ip address 192.168.45.17 255.255.255.240- ip nat inside

Interface g0/0.60 - encapsulation dot1q 60 - ip address 192.168.45.33 255.255.255.240- ip nat inside

Interface g0/0 no shutdown

Ip dhcp pool LAN - network 192.168.45.128 255.255.255.192 - default-router 192.168.45.129

Router rip - Version 2 - network 192.168.45.0 - passive-interface g0/0 - passive-interface g0/0.15- passive-interface g0/0.30- passive-interface g0/0.45- passive-interface g0/0.60 - exit

ip route 0.0.0.0 0.0.0.0 Serial 0/1/0

Router rip - default-information originate

Access-list 1 permit 192.168.45.0 0.0.0.255

Ip nat inside source static 192.168.45.66 64.100.32.58

ip nat pool NATPAT 64.100.32.56 64.100.32.57 netmask 255.255.255.252

Interface s0/0/0 - ip nat inside

Interface s0/0/1 - ip nat inside

Interface s0/1/0 - ip nat outside

Go to pc and change ip configuration to dhcp